CODE FOR ALL COMPONENTS INTERFACING (22/09/25)

import smbus

import time

import math

import RPi.GPIO as GPIO

from RPLCD.i2c import CharLCD

import serial

import webbrowser

import sys

# -------------------- MPU6050 Setup --------------------

MPU6050_ADDR = 0x68

PWR_MGMT_1 = 0x6B

ACCEL_XOUT_H = 0x3B

ACCEL_YOUT_H = 0x3D

ACCEL_ZOUT_H = 0x3F

bus = smbus.SMBus(1)

bus.write_byte_data(MPU6050_ADDR, PWR_MGMT_1, 0)

def read_raw_data(addr):

high = bus.read_byte_data(MPU6050_ADDR, addr)

low = bus.read_byte_data(MPU6050_ADDR, addr+1)

value = ((high << 8) | low)

if value > 32768:

value -= 65536

return value

# -------------------- MQ-3 Alcohol Sensor --------------------

SENSOR_PIN = 22

GPIO.setmode(GPIO.BCM)

GPIO.setup(SENSOR_PIN, GPIO.IN)

# -------------------- LCD Setup --------------------

lcd = CharLCD('PCF8574', 0x27)

# -------------------- GPS Setup --------------------

def convert_to_degrees(raw_value):

decimal_value = raw_value / 100.00

degrees = int(decimal_value)

mm_mmmm = (decimal_value - int(decimal_value)) / 0.6

position = degrees + mm_mmmm

position = "%.6f" % (position)

return position

def GPS_Info():

global NMEA_buff

global lat_in_degrees

global long_in_degrees

nmea_time = NMEA_buff[0]

nmea_latitude = NMEA_buff[1]

nmea_longitude = NMEA_buff[3]

print("NMEA Time:", nmea_time)

print("NMEA Latitude:", nmea_latitude, "NMEA Longitude:", nmea_longitude)

lat = float(nmea_latitude)

longi = float(nmea_longitude)

lat_in_degrees = convert_to_degrees(lat)

long_in_degrees = convert_to_degrees(longi)

ser = serial.Serial("/dev/serial0", baudrate=9600, timeout=1)

gpgga_info = "$GPGGA,"

GPGGA_buffer = 0

NMEA_buff = 0

lat_in_degrees = 0

long_in_degrees = 0

# -------------------- Main Loop --------------------

print("🚀 Smart Vehicle Safety System Running...\n")

try:

while True:

# ---- Accident Detection ----

acc_x = read_raw_data(ACCEL_XOUT_H) / 16384.0

acc_y = read_raw_data(ACCEL_YOUT_H) / 16384.0

acc_z = read_raw_data(ACCEL_ZOUT_H) / 16384.0

total_acc = math.sqrt(acc_x**2 + acc_y**2 + acc_z**2)

if total_acc > 2.0:

accident_status = "⚠️ Accident Detected!"

else:

accident_status = "✅ Safe"

# ---- Alcohol Detection ----

if GPIO.input(SENSOR_PIN) == GPIO.LOW:

alcohol_status = "🍷 Alcohol Detected!"

else:

alcohol_status = "No Alcohol"

# ---- GPS Data ----

received_data = ser.readline().decode('utf-8', errors='ignore')

GPGGA_data_available = received_data.find(gpgga_info)

if GPGGA_data_available > 0:

GPGGA_buffer = received_data.split("$GPGGA,", 1)[1]

NMEA_buff = (GPGGA_buffer.split(','))

GPS_Info()

gps_status = f"Lat:{lat_in_degrees} Long:{long_in_degrees}"

else:

gps_status = "GPS Searching..."

# ---- LCD Display ----

lcd.clear()

lcd.write_string(accident_status[:16])  # Line 1

lcd.crlf()

lcd.write_string(alcohol_status[:16])   # Line 2

# ---- Print Logs ----

print(f"{accident_status} | {alcohol_status} | {gps_status}")

time.sleep(1)

except KeyboardInterrupt:

print("Opening Google Maps with last location...")

map_link = f'http://maps.google.com/?q={lat_in_degrees},{long_in_degrees}'

webbrowser.open(map_link)

GPIO.cleanup()

sys.exit(0)