# smart_vehicle_monitoring.py

import time

from threading import Thread

import RPi.GPIO as GPIO

import serial

import smbus  # Often a dependency for I2C libraries

import cv2

import dlib

# GPIO setup

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

# --- CONFIGURATION ---

# Pin Configuration

MOTOR_IN1 = 20

MOTOR_IN2 = 21

BUZZER = 18

LED = 23

# Serial Device Paths

GPS_SERIAL_DEVICE = "/dev/ttyS0"

GSM_SERIAL_DEVICE = "/dev/ttyAMA0"

# Thresholds

ALCOHOL_THRESHOLD = 400

DROWSINESS_FRAME_THRESHOLD = 15  # Increased for more robust detection

ACCIDENT_THRESHOLD_X = 3.0

ACCIDENT_THRESHOLD_Y = 3.0

ACCIDENT_THRESHOLD_Z = 15.0

# Other

EMERGENCY_CONTACT_NUMBER = "+918122735200"

# --- HARDWARE INITIALIZATION ---

# Motor pins

GPIO.setup(MOTOR_IN1, GPIO.OUT)

GPIO.setup(MOTOR_IN2, GPIO.OUT)

# Buzzer and LED

BUZZER = 18

LED = 23

GPIO.setup(BUZZER, GPIO.OUT)

GPIO.setup(LED, GPIO.OUT)

# Shared state variable to control motor

motor_enabled = True

# Alcohol Sensor via MCP3008 (SPI)

import Adafruit_GPIO.SPI as SPI

import Adafruit_MCP3008

SPI_PORT = 0

SPI_DEVICE = 0

mcp = Adafruit_MCP3008.MCP3008(spi=SPI.SpiDev(SPI_PORT, SPI_DEVICE))

# MPU6050 (Accelerometer/Gyro)

from mpu6050 import mpu6050

mpu = mpu6050(0x68)

# LCD I2C Display

from lcd_i2c import LCD

lcd = LCD()

# GPS via UART

gps_serial = serial.Serial(GPS_SERIAL_DEVICE, baudrate=9600, timeout=1)

# GSM via UART

gsm_serial = serial.Serial(GSM_SERIAL_DEVICE, baudrate=9600, timeout=1)

# AI Camera Setup

# Note: Ensure 'haarcascade_eye.xml' is available.

# dlib's face detector is generally more robust than Haar cascades.

face_detector = dlib.get_frontal_face_detector()

eye_cascade = cv2.CascadeClassifier(cv2.data.haarcascades + "haarcascade_eye.xml")

def stop_motor():

GPIO.output(MOTOR_IN1, False)

GPIO.output(MOTOR_IN2, False)

def start_motor():

GPIO.output(MOTOR_IN1, True)

GPIO.output(MOTOR_IN2, False)

def read_alcohol_level():

return mcp.read_adc(0)

def read_gps_location():

"""

Reads and parses GPGGA sentences from the GPS module.

Converts NMEA format (DDMM.MMMMM) to Decimal Degrees.

"""

gps_serial.flushInput()

for _ in range(5): # Try reading a few lines

try:

line = gps_serial.readline().decode('utf-8', errors='ignore')

if '$GPGGA' in line:

parts = line.split(',')

if len(parts) > 5 and parts[2] and parts[4]:

# Latitude parsing

lat_nmea = parts[2]

lat_deg = int(lat_nmea[:2])

lat_min = float(lat_nmea[2:])

latitude = lat_deg + (lat_min / 60.0)

if parts[3] == 'S':

latitude *= -1

# Longitude parsing

lon_nmea = parts[4]

lon_deg = int(lon_nmea[:3])

lon_min = float(lon_nmea[3:])

longitude = lon_deg + (lon_min / 60.0)

if parts[5] == 'W':

longitude *= -1

return (latitude, longitude)

except (ValueError, IndexError):

continue # Ignore corrupted lines

return None

def send_sms(message):

"""Sends an SMS message using the GSM module."""

# Note: This is a basic implementation. A robust version would

# wait for 'OK' responses from the modem after each AT command.

gsm_serial.write(b'AT+CMGF=1\r')

time.sleep(1)

gsm_serial.write(f'AT+CMGS="{EMERGENCY_CONTACT_NUMBER}"\r'.encode())

time.sleep(1)

gsm_serial.write(message.encode())

gsm_serial.write(bytes([26]))  # Ctrl+Z

time.sleep(2)

def detect_accident():

accel_data = mpu.get_accel_data()

x, y, z = abs(accel_data['x']), abs(accel_data['y']), abs(accel_data['z'])

return x > ACCIDENT_THRESHOLD_X or y > ACCIDENT_THRESHOLD_Y or z > ACCIDENT_THRESHOLD_Z

def monitor_driver_activity():

global motor_enabled

cap = cv2.VideoCapture(0)

closed_eyes_counter = 0

while True:

ret, frame = cap.read()

if not ret:

time.sleep(0.1)

continue

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

faces = face_detector(gray)

for face in faces:

roi = gray[face.top():face.bottom(), face.left():face.right()]

eyes = eye_cascade.detectMultiScale(roi)

if len(eyes) == 0:

closed_eyes_counter += 1

else:

closed_eyes_counter = 0

if closed_eyes_counter >= DROWSINESS_FRAME_THRESHOLD:

if motor_enabled: # Trigger alert only once

print("Drowsiness Detected!")

motor_enabled = False

trigger_alert("Drowsiness Detected!", stop_vehicle=True, send_location=True)

closed_eyes_counter = 0

# A shorter sleep makes detection more responsive

time.sleep(0.1)

def trigger_alert(message, stop_vehicle=True

def main():

lcd.print("System Initialized")

driver_thread = Thread(target=monitor_driver_activity, daemon=True)

driver_thread.start()

while True:

alcohol = read_alcohol_level()

if alcohol > 400:

lcd.print("Alcohol Detected")

stop_motor()

send_sms("Alcohol detected. Vehicle stopped.")

else:

start_motor()

if detect_accident():

lcd.print("Accident Detected!")

GPIO.output(BUZZER, True)

GPIO.output(LED, True)

stop_motor()

location = read_gps_location()

if location:

msg = f"Accident! Location: https://maps.google.com/?q={location[0]},{location[1]}"

else:

msg = "Accident detected! Location not found."

send_sms(msg)

time.sleep(5)

GPIO.output(BUZZER, False)

GPIO.output(LED, False)

time.sleep(2)

if __name__ == "__main__":

try:

main()

except KeyboardInterrupt:

GPIO.cleanup()