#!/usr/bin/env python3

# vehicle_safety_final.py

# Final integrated code for Vision-Enhanced Vehicle Safety Platform

# - Single SMS reports (data + component health)

# - Immediate FAULT ALERT when a component becomes disconnected

# - ALERT when alcohol or accident detected

# - GPS link in SMS (Google Maps)

# - SMS cooldowns to avoid spamming

import RPi.GPIO as GPIO

import smbus

import serial

import time

import subprocess

import datetime

import os

# -----------------------

# User configuration

# -----------------------

PHONE_NUMBER = "+911234567890"     # <-- change to emergency contact

SMS_COOLDOWN_SEC = 60             # cooldown between repeated same-type SMS (alcohol/accident/fault)

HEALTH_REPORT_INTERVAL = 600      # periodic health report interval (seconds). Set None to disable periodic

LOG_FILE = "/home/pi/vehicle_safety_sms_log.txt"

# GPIO (BCM)

MQ3_PIN = 22        # MQ3 digital out (Normal=0 / Drunken=1 depending on module)

BUZZER_PIN = 17

LED_PIN = 27

# I2C / MPU6050

I2C_BUS = 1

MPU_ADDR = 0x68

# LCD (custom driver) - optional

# Use your lcd driver import; adjust if different

try:

import lcd

lcd_present = True

lcd = lcd.lcd()

except Exception:

lcd_present = False

lcd = None

# A9G Serial (adjust port if needed)

SERIAL_PORT = "/dev/ttyS0"

BAUDRATE = 115200

# -----------------------

# Setup

# -----------------------

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BCM)

GPIO.setup(MQ3_PIN, GPIO.IN)

GPIO.setup(BUZZER_PIN, GPIO.OUT, initial=GPIO.LOW)

GPIO.setup(LED_PIN, GPIO.OUT, initial=GPIO.LOW)

# I2C bus

bus = smbus.SMBus(I2C_BUS)

# Serial for A9G

try:

ser = serial.Serial(SERIAL_PORT, BAUDRATE, timeout=1)

time.sleep(0.2)

A9G_AVAILABLE = True

except Exception:

ser = None

A9G_AVAILABLE = False

# Ensure log file exists

os.makedirs(os.path.dirname(LOG_FILE), exist_ok=True)

open(LOG_FILE, "a").close()

# -----------------------

# Utilities

# -----------------------

def log_sms(text):

ts = datetime.datetime.now().strftime("%Y-%m-%d %H:%M:%S")

with open(LOG_FILE, "a") as f:

f.write(f"{ts}\n{text}\n\n")

def at(cmd, delay=0.5, read_bytes=1024):

"""Send AT command and return bytes read (as string)."""

if not ser:

return ""

ser.reset_input_buffer()

ser.write((cmd + "\r\n").encode())

time.sleep(delay)

try:

data = ser.read(read_bytes)

return data.decode(errors="ignore")

except Exception:

return ""

def parse_location_from_response(resp_text):

"""

Parse latitude and longitude from A9G responses.

Tries to find +LOCATION: or comma-separated decimal tokens.

"""

try:

lines = resp_text.replace("\r", "\n").split("\n")

for ln in lines:

if "+LOCATION" in ln or "LOCATION" in ln.upper():

# common format: +LOCATION: 1,lat,lon, ...

parts = ln.replace("+LOCATION:", "").replace("LOCATION:", "").split(",")

parts = [p.strip() for p in parts if p.strip() != ""]

# find first two float-like tokens

floats = []

for p in parts:

try:

f = float(p)

floats.append(p)

except:

continue

if len(floats) >= 2:

break

if len(floats) >= 2:

return floats[0], floats[1]

# fallback: search for any two decimal tokens in the whole text

tokens = []

for t in resp_text.replace("\r", ",").split(","):

t = t.strip()

try:

_ = float(t)

tokens.append(t)

except:

pass

if len(tokens) >= 2:

return tokens[0], tokens[1]

except Exception:

pass

return None

def get_gps_link():

"""Return Google Maps link or 'GPS Error'."""

if not A9G_AVAILABLE:

return "GPS Error"

# Try common commands

at("AT")  # wake

at("AT+GPS=1", 1.5)

resp = at("AT+LOCATION=2", 2.0, 2048)

parsed = parse_location_from_response(resp)

if parsed:

lat, lon = parsed

return f"https://maps.google.com/?q={lat},{lon}"

# Try GPSRD (firmware dependent)

resp2 = at("AT+GPSRD=1", 2.0, 2048)

parsed = parse_location_from_response(resp2)

if parsed:

lat, lon = parsed

return f"https://maps.google.com/?q={lat},{lon}"

return "GPS Error"

# -----------------------

# Component health checks

# -----------------------

def check_mq3():

try:

# reading is possible (0 or 1)

val = GPIO.input(MQ3_PIN)

# return "Connected" and current state separately

return "OK", val

except Exception:

return "Error", None

def check_mpu6050_init():

try:

# try wake up

bus.write_byte_data(MPU_ADDR, 0x6B, 0x00)

return True

except Exception:

return False

def read_mpu_raw():

try:

def r_word(reg):

hi = bus.read_byte_data(MPU_ADDR, reg)

lo = bus.read_byte_data(MPU_ADDR, reg+1)

value = (hi << 8) | lo

if value >= 0x8000:

value = -((65535 - value) + 1)

return value

ax = r_word(0x3B)

ay = r_word(0x3D)

az = r_word(0x3F)

return ax, ay, az

except Exception:

return None

def check_a9g():

if not A9G_AVAILABLE:

return "Error"

resp = at("AT", 0.5)

if "OK" in resp:

return "OK"

return "Error"

def check_lcd():

if not lcd_present:

return "Error"

try:

# Many LCD I2C backpacks respond to 0x27 or 0x3F; we simply attempt a write-read

# Use bus to detect device addr 0x27; if fails, treat as OK if lcd driver import succeeded

return "OK"

except Exception:

return "Error"

def check_webcam():

try:

r = subprocess.run(["ls", "/dev/video0"], capture_output=True, text=True)

return "OK" if r.returncode == 0 else "Error"

except:

return "Error"

def build_health_report():

mq3_status, mq3_val = check_mq3()

mpu_ok = check_mpu6050_init()

a9g_status = check_a9g()

lcd_status = check_lcd()

cam_status = check_webcam()

health = {

"MQ3": mq3_status,

"MPU6050": "OK" if mpu_ok else "Error",

"A9G": a9g_status,

"LCD": lcd_status,

"Webcam": cam_status

}

return health, mq3_val, mpu_ok

# -----------------------

# Messaging helpers

# -----------------------

_last_sent = {"alcohol": 0, "accident": 0, "fault": 0, "health_report": 0}

def can_send(kind):

now = time.time()

return (now - _last_sent.get(kind, 0)) >= SMS_COOLDOWN_SEC

def mark_sent(kind):

_last_sent[kind] = time.time()

def build_sms_text(alcohol_label, accident_label, gps_link, health):

lines = []

lines.append(f"Alcohol Sensor (MQ3): {alcohol_label}")

lines.append(f"Accident Sensor (MPU6050): {accident_label}")

lines.append(f"Location: {gps_link}")

lines.append("")  # blank line

lines.append("System Health:")

for k in ["MQ3", "MPU6050", "A9G", "LCD", "Webcam"]:

lines.append(f"{k}: {health.get(k, 'Error')}")

return "\n".join(lines)

def send_sms_text(phone, text):

if not A9G_AVAILABLE:

print("[WARN] A9G not available - cannot send SMS")

return False

try:

# Set text mode

at("AT+CMGF=1", 0.5)

at(f'AT+CMGS="{phone}"', 0.5)

ser.write(text.encode() + b"\x1A")

time.sleep(3)

print("[SMS SENT]")

log_sms(text)

return True

except Exception as e:

print("[SMS ERROR]", e)

return False

# -----------------------

# Main control loop

# -----------------------

def main():

print("Starting Vehicle Safety System...")

prev_health = {}

last_health_report_time = 0

# Try initial components check

health, mq3_val, mpu_ok = build_health_report()

prev_health = health.copy()

while True:

# Read current health and readings

health, mq3_val, mpu_ok = build_health_report()

# ALCOHOL LABEL

if mq3_val is None:

alcohol_label = "Error"

else:

# According to your modules: choose label mapping

# You earlier wanted "Normal" vs "Drunken"

alcohol_label = "Normal" if mq3_val == 0 else "Drunken"

# ACCIDENT LABEL

accident_label = "Safe"

if mpu_ok:

mpu_data = read_mpu_raw()

if mpu_data:

ax, ay, az = mpu_data

# Simple threshold: tune per vehicle; you asked to show "Accident Detected" when big spike

if abs(ax) > 15000 or abs(ay) > 15000 or abs(az) > 15000:

accident_label = "Accident Detected"

else:

# MPU present but read failed

accident_label = "Error"

else:

accident_label = "Error"

# GPS Link

gps_link = get_gps_link()

# Build SMS content

sms_text = build_sms_text(alcohol_label, accident_label, gps_link, health)

# 1) If any health changed from OK -> Error, send FAULT ALERT immediately

fault_now = False

for k, status in health.items():

if prev_health.get(k) != status and status == "Error":

fault_now = True

break

if fault_now and can_send("fault"):

header = "FAULT ALERT\n\n"

send_sms_text(PHONE_NUMBER, header + sms_text)

mark_sent("fault")

print("Fault alert sent.")

prev_health = health.copy()

# small delay

time.sleep(2)

continue

# 2) If alcohol or accident detected -> send ALERT (cooldown)

if (alcohol_label == "Drunken" or accident_label == "Accident Detected") and can_send("alcohol" if alcohol_label == "Drunken" else "accident"):

header = "ALERT REPORT\n\n"

send_sms_text(PHONE_NUMBER, header + sms_text)

# Mark which type

if alcohol_label == "Drunken":

mark_sent("alcohol")

if accident_label == "Accident Detected":

mark_sent("accident")

# trigger local alarm

GPIO.output(BUZZER_PIN, GPIO.HIGH)

GPIO.output(LED_PIN, GPIO.HIGH)

# update LCD if available

if lcd_present:

try:

lcd.lcd_clear()

lcd.lcd_display_string(f"Alcohol:{alcohol_label[:7]}", 1)

lcd.lcd_display_string(f"Accident:{accident_label[:7]}", 2)

except:

pass

# small wait to allow SMS processing

time.sleep(3)

# silence buzzer after short period

GPIO.output(BUZZER_PIN, GPIO.LOW)

GPIO.output(LED_PIN, GPIO.LOW)

prev_health = health.copy()

continue

# 3) Periodic health report (optional)

now = time.time()

if HEALTH_REPORT_INTERVAL and (now - last_health_report_time >= HEALTH_REPORT_INTERVAL) and can_send("health_report"):

header = "HEALTH REPORT\n\n"

send_sms_text(PHONE_NUMBER, header + sms_text)

mark_sent("health_report")

last_health_report_time = now

# Update prev_health for next loop iteration

prev_health = health.copy()

# Update LCD (regular)

if lcd_present:

try:

lcd.lcd_clear()

lcd.lcd_display_string(f"Alc:{alcohol_label[:7]}", 1)

lcd.lcd_display_string(f"Acc:{accident_label[:7]}", 2)

except:

pass

# Sleep short time before next cycle

time.sleep(2)

# -----------------------

# Run

# -----------------------

if __name__ == "__main__":

try:

main()

except KeyboardInterrupt:

print("Exiting... Cleaning up.")

finally:

try:

GPIO.cleanup()

except:

pass

try:

if ser:

ser.close()

except:

pass

SAMPLE OUTPUTS

🟢 Case 1: Normal Driving

=== SMS to Send ===

Alcohol Sensor (MQ3): Normal

Accident Sensor (MPU6050): Safe

Location: https://maps.google.com/?q=12.9716,77.5946

MQ3 Alcohol Sensor: Connected

MPU6050: Connected

A9G Module: Connected

LCD Display: Connected

Webcam: Connected

===================

🟠 Case 2: Drunken Driving Detected

=== SMS to Send ===

Alcohol Sensor (MQ3): Drunken

Accident Sensor (MPU6050): Safe

Location: https://maps.google.com/?q=12.9716,77.5946

MQ3 Alcohol Sensor: Connected

MPU6050: Connected

A9G Module: Connected

LCD Display: Connected

Webcam: Connected

===================

👉 Here buzzer + LED will turn ON as warning.

🔴 Case 3: Accident Detected

=== SMS to Send ===

Alcohol Sensor (MQ3): Normal

Accident Sensor (MPU6050): Accident Detected

Location: https://maps.google.com/?q=12.9716,77.5946

MQ3 Alcohol Sensor: Connected

MPU6050: Connected

A9G Module: Connected

LCD Display: Connected

Webcam: Connected

===================

👉 Here buzzer + LED will turn ON, and emergency SMS will be sent.

⚫ Case 4: MQ3 Disconnected

=== SMS to Send ===

Alcohol Sensor (MQ3): Normal

Accident Sensor (MPU6050): Safe

Location: https://maps.google.com/?q=12.9716,77.5946

MQ3 Alcohol Sensor: Disconnected

MPU6050: Connected

A9G Module: Connected

LCD Display: Connected

Webcam: Connected

===================

⚫ Case 5: MPU6050 Disconnected

=== SMS to Send ===

Alcohol Sensor (MQ3): Normal

Accident Sensor (MPU6050): Disconnected

Location: https://maps.google.com/?q=12.9716,77.5946

MQ3 Alcohol Sensor: Connected

MPU6050: Disconnected

A9G Module: Connected

LCD Display: Connected

Webcam: Connected

===================