Updated code for the project(30/07/2025)

# Raspberry Pi 4-Based Smart Vehicle Safety System

# Features: Alcohol Detection, Accident Alert, GPS Tracking, GSM Alert, AI Camera Monitoring

import RPi.GPIO as GPIO

import time

import serial

import smbus2

from mpu6050 import mpu6050

from lcd_i2c import LCD

from picamera import PiCamera

# ---------------- GPIO SETUP ----------------

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

# MQ3 Alcohol Sensor (Digital Output)

MQ3_PIN = 24

GPIO.setup(MQ3_PIN, GPIO.IN)

# Motor Control - L298N

MOTOR_IN1 = 20

MOTOR_IN2 = 21

GPIO.setup(MOTOR_IN1, GPIO.OUT)

GPIO.setup(MOTOR_IN2, GPIO.OUT)

# Buzzer & LED

BUZZER = 18

LED = 23

GPIO.setup(BUZZER, GPIO.OUT)

GPIO.setup(LED, GPIO.OUT)

# LCD Display

lcd = LCD()  # Assumes I2C address 0x27

# MPU6050 - Accelerometer

mpu = mpu6050(0x68)

# GPS via UART0 (Pi's default serial)

gps_serial = serial.Serial("/dev/serial0", baudrate=9600, timeout=1)

# GSM via secondary UART (GPIO16/17 must be enabled in config.txt)

gsm_serial = serial.Serial("/dev/ttyS0", baudrate=9600, timeout=1)

# Pi Camera

camera = PiCamera()

# ---------------- FUNCTION DEFINITIONS ----------------

def stop_motor():

GPIO.output(MOTOR_IN1, GPIO.LOW)

GPIO.output(MOTOR_IN2, GPIO.LOW)

def start_motor():

GPIO.output(MOTOR_IN1, GPIO.HIGH)

GPIO.output(MOTOR_IN2, GPIO.LOW)

def send_sms(message):

gsm_serial.write(b'AT+CMGF=1\r')

time.sleep(1)

gsm_serial.write(b'AT+CMGS="+918122735100"\r')  # Replace with your number

time.sleep(1)

gsm_serial.write(message.encode())

gsm_serial.write(bytes([26]))  # Ctrl+Z to send

time.sleep(3)

def get_gps_location():

gps_data = gps_serial.readline().decode('utf-8', errors='ignore')

if "$GPGGA" in gps_data:

parts = gps_data.split(",")

if parts[2] and parts[4]:

lat = float(parts[2][:2]) + float(parts[2][2:]) / 60

lon = float(parts[4][:3]) + float(parts[4][3:]) / 60

return f"https://maps.google.com/?q={lat:.6f},{lon:.6f}"

return "Location Not Found"

def detect_accident():

accel = mpu.get_accel_data()

magnitude = (accel['x']**2 + accel['y']**2 + accel['z']**2) ** 0.5

return magnitude > 25  # adjust threshold if needed

def monitor_driver():

# Placeholder for future ML driver behavior monitoring (OpenCV, dlib, etc.)

pass

# ---------------- MAIN LOOP ----------------

lcd.lcd_display_string("System Booting...", 1)

time.sleep(2)

lcd.lcd_clear()

try:

while True:

alcohol_detected = GPIO.input(MQ3_PIN)

gps_link = get_gps_location()

if alcohol_detected:

lcd.lcd_display_string("Alcohol Detected", 1)

stop_motor()

GPIO.output(LED, GPIO.HIGH)

send_sms("[ALERT] Alcohol detected. Motor stopped.\nLocation: " + gps_link)

else:

lcd.lcd_display_string("Safe to Drive", 1)

start_motor()

GPIO.output(LED, GPIO.LOW)

if detect_accident():

lcd.lcd_display_string("Accident Alert!", 1)

GPIO.output(BUZZER, GPIO.HIGH)

stop_motor()

send_sms("[EMERGENCY] Accident detected!\nLocation: " + gps_link)

time.sleep(5)

GPIO.output(BUZZER, GPIO.LOW)

monitor_driver()  # Extend later with AI

time.sleep(2)

except KeyboardInterrupt:

GPIO.cleanup()

lcd.lcd_clear()

print("System stopped by user.")