# Vehicle Monitoring and Safety System - Raspberry Pi (Final Version)

# 1. Code Functionality Table

| Section | Function | Description |
| --- | --- | --- |
| Imports | import RPi.GPIO, cv2, serial, etc. | Loads libraries for GPIO, camera, sensors, serial communication. |
| GPIO Setup | GPIO.setup(...) | Configures Pi pins for motor, buzzer, LED. |
| MCP3008 (ADC) | Adafruit_MCP3008 | Reads analog MQ3 alcohol sensor via SPI. |
| MPU6050 | mpu6050(0x68) | Initializes accelerometer/gyroscope over I2C. |
| LCD | lcd = LCD() | Handles I2C LCD printing of messages. |
| GPS & GSM | serial.Serial(...) | Sets UART for GPS & GSM modules. |
| AI Camera | cv2 + dlib | Face & eye detection using OpenCV and dlib. |
| Motor Control | start_motor(), stop_motor() | Controls motor driver pins. |
| Alcohol Detection | read_alcohol_level() | Reads alcohol sensor and checks threshold. |
| GPS Location | read_gps_location() | Parses latitude/longitude from GPS NMEA. |
| Send SMS | send_sms(msg) | Uses AT commands to send emergency SMS. |
| Accident Detection | detect_accident() | Reads MPU6050 values to detect accident. |
| Driver Monitoring | monitor_driver_activity() | Detects drowsiness using camera. |
| Main Loop | while True: | Main system loop: alcohol, accident, AI monitor. |

# 2. Raspberry Pi GPIO Circuit Connections

| Component | Pin Type | Raspberry Pi GPIO Pin | Notes |
| --- | --- | --- | --- |
| MQ3 Alcohol Sensor | Analog via MCP3008 | MCP3008 CH0 → SPI0 | Reads analog value using ADC. |
| MCP3008 ADC | SPI | MOSI: GPIO10, MISO: GPIO9, CLK: GPIO11, CS: GPIO8 | Analog-Digital converter for MQ3. |
| MPU6050 | I2C | SDA: GPIO2, SCL: GPIO3 | Motion sensor for accident detection. |
| GPS (NEO-6M) | UART | TX → GPIO15, RX → GPIO14 | Receives GPS data from NEO-6M. |
| GSM (SIM800L) | UART | TX → GPIO14, RX → GPIO15 | Sends SMS using AT commands. |
| LCD Display (I2C) | I2C | SDA: GPIO2, SCL: GPIO3 | Shows system messages. |
| Motor Driver (L298N) | Digital Output | IN1: GPIO20, IN2: GPIO21 | Controls forward/back motor. |
| Buzzer | Digital Output | GPIO18 | Buzzer for alert signals. |
| LED | Digital Output | GPIO23 | Visual accident/alcohol alert. |
| AI Camera | USB or CSI | USB0 or Camera Port | Used for drowsiness/driver monitoring. |

# 3. Functions of Individual Components

| Component | Type | Function |
| --- | --- | --- |
| MQ3 Alcohol Sensor | Gas Sensor | Detects alcohol vapors from driver’s breath. |
| MCP3008 | ADC | Converts analog MQ3 output to digital for Raspberry Pi. |
| MPU6050 | Accelerometer + Gyroscope | Detects sudden motion or collision (accident). |
| GPS (NEO-6M) | Positioning Module | Provides latitude and longitude of vehicle. |
| GSM (SIM800L) | Communication Module | Sends SMS alerts with GPS and status. |
| LCD Display (I2C) | Output Display | Displays system status and alerts. |
| L298N Motor Driver | Motor Controller | Controls the motor ON/OFF and direction. |
| Buzzer | Audio Alert | Emits sound during alerts (alcohol/accident/drowsiness). |
| LED | Visual Indicator | Lights up during critical warnings. |
| AI Camera (USB/Pi Cam) | Computer Vision Module | Monitors driver’s face for drowsiness or distraction. |