🔹 Step 1: Test MQ3 Alcohol Sensor (GPIO24 → LED + Buzzer)

import RPi.GPIO as GPIO

import time

# Pin setup

MQ3_PIN = 24

LED_PIN = 23

BUZZER_PIN = 18

GPIO.setmode(GPIO.BCM)

GPIO.setup(MQ3_PIN, GPIO.IN)

GPIO.setup(LED_PIN, GPIO.OUT)

GPIO.setup(BUZZER_PIN, GPIO.OUT)

print("MQ3 Sensor Test - Blow near sensor with alcohol")

try:

while True:

if GPIO.input(MQ3_PIN) == 0:  # 0 = Alcohol detected

print("Alcohol detected!")

GPIO.output(LED_PIN, 1)

GPIO.output(BUZZER_PIN, 1)

else:

print("No alcohol")

GPIO.output(LED_PIN, 0)

GPIO.output(BUZZER_PIN, 0)

time.sleep(1)

except KeyboardInterrupt:

GPIO.cleanup()

🔹 Step 2: Test LED & Buzzer (GPIO23, GPIO18)

import RPi.GPIO as GPIO

import time

LED_PIN = 23

BUZZER_PIN = 18

GPIO.setmode(GPIO.BCM)

GPIO.setup(LED_PIN, GPIO.OUT)

GPIO.setup(BUZZER_PIN, GPIO.OUT)

print("Testing LED & Buzzer")

try:

while True:

GPIO.output(LED_PIN, 1)

GPIO.output(BUZZER_PIN, 1)

print("ON")

time.sleep(1)

GPIO.output(LED_PIN, 0)

GPIO.output(BUZZER_PIN, 0)

print("OFF")

time.sleep(1)

except KeyboardInterrupt:

GPIO.cleanup()

🔹 Step 3: Test L298N Motor Driver (GPIO20, GPIO21)

import RPi.GPIO as GPIO

import time

IN1 = 20

IN2 = 21

GPIO.setmode(GPIO.BCM)

GPIO.setup(IN1, GPIO.OUT)

GPIO.setup(IN2, GPIO.OUT)

print("Motor Driver Test")

try:

while True:

print("Forward")

GPIO.output(IN1, 1)

GPIO.output(IN2, 0)

time.sleep(2)

print("Reverse")

GPIO.output(IN1, 0)

GPIO.output(IN2, 1)

time.sleep(2)

except KeyboardInterrupt:

GPIO.cleanup()

🔹 Step 4: Test MPU6050 (I2C, addr=0x68)

import smbus

import time

bus = smbus.SMBus(1)

MPU_ADDR = 0x68

# Wake up MPU6050

bus.write_byte_data(MPU_ADDR, 0x6B, 0)

def read_word(adr):

high = bus.read_byte_data(MPU_ADDR, adr)

low = bus.read_byte_data(MPU_ADDR, adr+1)

val = (high << 8) + low

if val >= 0x8000:

return -((65535 - val) + 1)

else:

return val

print("MPU6050 Test - Move module to see values")

try:

while True:

acc_x = read_word(0x3B)

acc_y = read_word(0x3D)

acc_z = read_word(0x3F)

print(f"X={acc_x}, Y={acc_y}, Z={acc_z}")

time.sleep(0.5)

except KeyboardInterrupt:

pass

🔹 Step 5: Test A9G GSM/GPS (UART)

import serial

import time

ser = serial.Serial("/dev/serial0", 9600, timeout=1)

time.sleep(2)

print("A9G GSM/GPS Test")

ser.write(b'AT\r\n')   # Test AT

time.sleep(1)

print(ser.read(100).decode())

ser.write(b'AT+GPS=1\r\n')  # Enable GPS

time.sleep(2)

print(ser.read(200).decode())

# Send SMS (replace with your number)

# ser.write(b'AT+CMGF=1\r\n')

# time.sleep(1)

# ser.write(b'AT+CMGS="+918122735100"\r\n')

# time.sleep(1)

# ser.write(b'Hello from A9G\x1A')

# time.sleep(2)

🔹 Step 6: Test I2C LCD (GPIO2=SDA, GPIO3=SCL)

from RPLCD.i2c import CharLCD

import time

lcd = CharLCD('PCF8574', 0x27)  # Use 0x3F if 0x27 doesn’t work

lcd.clear()

lcd.write_string("LCD Test OK")

time.sleep(2)

lcd.clear()

lcd.write_string("Hello World")

🔹 Step 7: Test Webcam (USB/CSI)

import cv2

cap = cv2.VideoCapture(0)

if not cap.isOpened():

print("Camera not detected")

else:

print("Press Q to quit")

while True:

ret, frame = cap.read()

if ret:

cv2.imshow("Webcam Test", frame)

if cv2.waitKey(1) & 0xFF == ord('q'):

break

cap.release()

cv2.destroyAllWindows()

✅ With these scripts, you can check each module separately.